- Model 500S ‘Shrike’

- Type Certificate:	6A1

Date Certificated:	March 15th 1968 (NOTE: was Certificated in both the Normal and Utility categories)

Seats:	7	(2 at +94, 2 at +128, 3 at +168)

8 	(when modified in accordance with Aero Commander Drawing 6100023)

Number built:	316 (see Notes)

Certificated by:	Aero Commander Division, North American Rockwell Corporation

Serial numbers 1755-1 through 1876-49 & 3050 through 3162 	(162 aircraft)

Commander Aircraft Division, Rockwell International Corporation

Serial numbers 3163 through 3206 	(44 aircraft)

- General Aviation Division, Rockwell International Corporation

Serial numbers 3207 through 3282 & 3290 through 3323 	(110 aircraft)

Engines:	Lycoming IO-540-E1B5

Serial numbers 1755-1 through 1876-49 & 3050 through 3282 	(282 aircraft)

Serial numbers 3290 through 3323 	(34 aircraft)

Propellers (hub / blade): 	Hartzell HC-A3VK-2A / V8433-4

Serial numbers 1755-1 through 1876-49 & 3050 through 3053 	(53 aircraft)

Hartzell HC-C3YR-2 / C8468-6R

Serial numbers 3054 through 3123	(70 aircraft)

Hartzell HC-C3YR-2F / FC8468-6R

Serial numbers 3124 through 3278, plus 3300	(156 aircraft)

Hartzell HC-C3YR-2UF / FC8468-6R

Serial numbers 3279 through 3282, 3290 through 3299 & 3301 through 3323	(37 aircraft)

Fuel:	100/130 minimum grade aviation gasoline

Fuel consumption:	13.0 U.S. gallons per engine per hour at 2,400r.p.m.

Fuel capacity:	159.6 U.S. gallons (604 litres) total	[5 inter-connected tanks]

156.0 U.S. gallons (591 litres) usable

Oil capacity:	6 U.S. gallons (22.71 litres) total (3 U.S. gallons each engine)

Note:

The Engines and Propellers itemised above are those that were installed prior to delivery from the factory. See both the ‘Engines’ and ‘Propellers’ tables which follow, for other installations which are listed on the Type Certificate Data Sheet (TCDS), have been used for Experimental purposes, or are applicable under a Supplemental Type Certificate (STC).

| Dimensions (1): |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet/Inches | Inches | Meters | Remarks |
| Wing - span | 49' 0.56" | 588.56 | 14.949 |  |
| Wing - chord at root | 8' 4.00" | 100.00 | 2.540 |  |
| Wing - chord at tip | 2' 1.42" | 25.42 | 0.646 |  |
| Wing - Mean Aerodynamic Chord | 5' 10.05" | 70.05 | 1.779 |  |
| Wing - ground to wingtip height | 7' 2.60" | 86.60 | 2.200 |  |
| Wing - area, inc. Flaps & Ailerons | 255.00 sq.ft. |  | 23.690 m2 |  |
| Wing - total Flaps area | 21.20 sq.ft. |  | 1.970 m2 |  |
| Wing - total Ailerons area | 20.52 sq.ft. |  | 1.906 m2 |  |
| Wing - Airfoil section | NACA 23012 modified |  |  | tapers to NACA 23015 from stn. 257 |
| Wing - Aspect Ratio | 9.45 |  |  |  |
| Wing - Taper Ratio | 3.93 |  |  |  |
| Wing - dihedral | 4 degrees |  |  |  |
| Wing - incidence at root | 3 degrees |  |  |  |
| Wing - incidence at tip | minus 3.5 degrees |  |  | Maintenance Manual omits the ‘minus’ |
| Wing - sweep forward at trailing edge | 14 degrees |  |  | Maintenance Manual = 3 degrees |
| Wing - sweep angle at 25% chord | minus 3.567 degrees |  |  | Drawings 5010221 & J010247 = -3o 32' |
| Fin - height above fuselage top | 8' 4.00" | 100.00 | 2.540 | s/ns 3076 & up - 102.2" including beacon |
| Fin - chord at root | 6' 4.344" | 76.344 | 1.939 |  |
| Fin - chord at tip | 3' 0.55" | 36.55 | 0.928 |  |
| Fin - Mean Aerodynamic Chord | 4' 10.785" | 58.785 | 1.493 |  |
| Fin - total overall area | 39.20 sq.ft. |  | 3.642 m2 |  |
| Fin - fixed surface area | 23.82 sq.ft. |  | 2.213 m2 |  |
| Fin - Rudder area | 15.38 sq.ft. |  | 1.429 m2 | including trim tab |
| Fin - Airfoil section | NACA 0009 |  |  | measured normal to 50% chord line |
| Fin - Aspect Ratio | 1.77 |  |  |  |
| Fin - Taper Ratio | 2.09 |  |  | Drawings 5010221 & J010247 = 2.15 |
| Fin - sweep angle at 25% chord line | 20.372 degrees |  |  | Drawings 5010221 & J010247 = 20o 20' |
| Tailplane - span | 16' 8.95" | 200.95 | 5.104 |  |
| Tailplane - chord at root | 4' 9.75" | 57.75 | 1.467 |  |
| Tailplane - chord at tip | 1' 7.03" | 19.03 | 0.483 |  |
| Tailplane - Mean Aerodynamic Chord | 3' 5.644" | 41.644 | 1.058 |  |
| Tailplane - ground to tip height | 7' 7.30" | 91.30 | 2.319 |  |
| Tailplane - total overall area | 54.40 sq.ft. |  | 5.054 m2 |  |
| Tailplane - fixed surface area | 33.86 sq.ft. |  | 3.146 m2 |  |
| Tailplane - total Elevators area | 20.54 sq.ft. |  | 1.908 m2 | including trim tab |
| Tailplane - Airfoil section | NACA 0011 (chordwise) |  |  |  |
| Tailplane - Aspect Ratio | 5.155 |  |  |  |
| Tailplane - Taper Ratio | 3.035 |  |  | Drawings 5010221 & J010247 = 3.01 |
| Tailplane - dihedral | 10 degrees |  |  |  |
| Tailplane - angle of incidence | 0.75 degrees |  |  |  |
| Tailplane - sweep angle at 25% chord | 5.372 degrees |  |  | Drawings 5010221 & J010247 = 5o 20' |
| Overall length - normal nose | 35' 5.50" | 425.50 | 10.808 | standard - s/ns 1755 thru 1876 |
| Overall length - Miller nose STC | 36' 8.16" | 440.16 | 11.180 | ? |
| Overall length - factory fiberglass nose | 36' 7.25" | 439.25 | 11.157 | standard - s/ns 3050 thru 3075 (?) |
| Overall length - factory metal nose | 36' 9.72" | 441.72 | 11.220 | standard - s/ns 3076 and up |
| Fuselage length - normal nose | 35' 1.25" | 421.25 | 10.700 | standard - s/ns 1755 thru 1876 |
| Overall height (gear static) 1755-3075 | 14' 6.00" | 174.00 | 4.420 | does not include any fin-top beacon |
| Overall height (gear static) 3076 & up | 14' 8.20" | 176.20 | 4.475 | includes 2.20" for the 'tear-drop' beacon |
| Height - ground to fuselage top | 6' 2.00" | 74.00 | 1.880 |  |
| Ground to fuselage clearance | 9.00" | 9.00 | 0.229 | Drawings 5010221 & J010247 = 9.20" when struts static and 5.70" when struts compressed |
| Maximum external fuselage height | 5' 5.00" | 65.00 | 1.651 |  |
| Maximum external fuselage width | 4' 8.00" | 56.00 | 1.422 |  |

| Dimensions (2): |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feet/Inches | Inches | Meters | Remarks |
| Wheel track (tread) | 12' 11.00" | 155.00 | 3.937 |  |
| Wheelbase (nose gear to main gear) | 13' 11.81" | 167.81 | 4.262 |  |
| Main gear deflection - below static | 0' 4.98" | 4.98 | 0.126 |  |
| Main gear deflection - above static | 0' 3.77" | 3.77 | 0.096 |  |
| Nose gear deflection - below static | 0' 5.19" | 5.19 | 0.132 |  |
| Nose gear deflection - above static | 0' 2.31" | 2.31 | 0.059 |  |
| Minimum ground turning distance | 77' 0.00" | 924.00 | 23.470 |  |
| Cabin - internal length (total) | 10' 7.50" | 127.50 | 3.239 |  |
| Cabin - internal height | 4' 4.00" | 52.00 | 1.321 |  |
| Cabin - internal width | 4' 4.00" | 52.00 | 1.321 |  |
| Cabin - volume | 160 cu.ft. |  | 4.531 m3 | disregarding all references to 177 cu.ft. |
| Main cabin door - height | 3' 7.00" | 43.00 | 1.092 |  |
| Main cabin door - width | 2' 4.00" | 28.00 | 0.711 |  |
| Main cabin door - height to sill | 1' 6.50" | 18.50 | 0.470 |  |
| Forward cabin door - height | 3' 9.00" | 45.00 | 1.143 |  |
| Forward cabin door - width | 1' 11.00" | 23.00 | 0.584 |  |
| Forward cabin door - height to sill | 1' 6.50" | 18.50 | 0.470 |  |
| Baggage compartment - length | 2' 7.00" | 31.00 | 0.787 |  |
| Baggage compartment - height | 3' 5.00" | 41.00 | 1.041 |  |
| Baggage compartment - width | 3' 11.00" | 47.00 | 1.194 |  |
| Baggage compartment - volume | 32 cu.ft. |  | 0.906 m3 |  |
| Baggage door - height | 1' 11.50" | 23.50 | 0.597 |  |
| Baggage door - width | 1' 7.50" | 19.50 | 0.495 |  |
| Baggage door - height to sill | 1' 9.70" | 21.70 | 0.551 |  |
| Drawing No. 5010221 for s/ns 1755 through 3075; Drawing No. J010247 for s/ns 3076 & up (the ‘1970 Model’). |  |  |  |  |

| Weights & Balance: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | lb | kg |  |  |
| Ramp Weight | 6,750 | 3,062 |  |  |
| MTOW | 6,750 | 3,062 |  |  |
|  | 7,800 | 3,538 | under STC SA2478SW (IO-720 series engines) |  |
| MLW | 6,750 | 3,062 |  |  |
|  | 7,800 | 3,538 | under STC SA2478SW (IO-720 series engines) |  |
| MZFW | 6,250 | 2,835 |  |  |
| Empty Weight (typical) | 4,635 | 2,102 | Factory Empty Weight, with standard equipment |  |
| Useful load | 2,115 | 959 |  |  |
| Basic Operating Weight |  |  |  |  |
| Payload |  |  |  |  |
| Maximum baggage | 500 | 227 |  |  |
| Maximum Wing Loading | 26.47 lb./sq.ft. | 129.24 kg/m2 | 6,750lb. / 255.00 sq.ft. |  |
|  | 30.59 lb./sq.ft. | 149.34 kg/m2 | 7,800lb. / 255.00 sq.ft. |  |
| Maximum Power Loading | 11.64 lb./h.p. | 5.28 kg/h.p. | 7.08 kg/kW | 6,750lb. / 580 h.p. |
|  | 9.64 lb./h.p. | 4.37 kg/h.p. | 5.87 kg/kW | with TIO-540-J2B: 6,750lb. / 700 h.p. |
|  | 9.75 lb./h.p. | 4.42 kg/h.p. | 5.93 kg/kW | with IO-720-B1B: 7,800lb. / 800 h.p. |
| Flight Load Factor | 4.40 G’s |  |  |  |
| C.G. range | (+166.0) to (+174.4) - gear extended (without Service Bulletin No. 128 or 129 installed) |  |  |  |
| s/n 1755 - 3155 at 5,600lb | (+161.8) to (+174.5) - gear extended (with Service Bulletin No. 128 or 129 installed) |  |  |  |
| s/n 1755 - 3155 at 6,750lb | (+166.0) to (+173.0) - gear extended (with Service Bulletin No. 128 or 129 installed) |  |  |  |
| s/n 3156 - 3323 at 5,600lb | (+161.8) to (+174.5) - gear extended |  |  |  |
| s/n 3156 - 3323 at 6,750lb | (+166.0) to (+173.0) - gear extended |  |  |  |
| Effect of retracting gear | +10,073 in./lb. |  |  |  |
| Empty Weight C.G. range | None |  |  |  |
| Datum | Located 152 inches (12' 8") forward of wing leading edge at center section. |  |  |  |
| Levelling means | Longitudinal | Top of fuselage centerline aft of wing trailing edge. |  |  |
|  | Lateral | Transverse beams at front or rear of baggage compartment floor. |  |  |

| Control Surface Movements: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | o up | o down | o right | o left | Remarks |
| Aileron | 23 2 | 15 2 |  |  |  |
| Flaps |  | 40 2 |  |  |  |
| Elevator | 30 +1/-0 | 10 +2/-0 |  |  |  |
| Elevator tab | 2.5 +2/-0.5 | 26 +2/-0 |  |  |  |
| Rudder |  |  | 20 +2/-0 | 20 +2/-0 |  |
| Rudder tab |  |  | 26 +2/-0 | 26 +2/-0 |  |

| Performance: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Feet | Meters |  | Remarks |
| Takeoff distance, over 50ft obstacle |  | 1,915 | 584 |  |  |
| Takeoff run |  |  |  |  |  |
| Short-field takeoff, over 50 ft obstacle |  |  |  |  |  |
| Short-field takeoff run |  |  |  |  |  |
| Landing distance, over 50ft obstacle |  | 2,235 | 681 |  |  |
| Landing run |  |  |  |  |  |
| Short-field landing distance, over 50ft obstacle |  |  |  |  |  |
| Short-field landing run |  |  |  |  |  |
| Maximum Operating Altitude |  |  |  |  | FAA Operational Ceiling |
| Service ceiling |  | 19,400 | 5,913 |  |  |
| Single-engine ceiling |  | 8,000 | 2,438 |  | Jane’s = 6,500’ |
|  |  | fpm | mpm |  |  |
| Initial Rate of Climb |  |  |  |  |  |
| Maximum Rate of Climb at Sea Level |  | 1,340 | 408 |  |  |
| Maximum Rate of Climb at Sea Level, single engine |  | 266 | 81 |  |  |
|  |  | nm | sm | km |  |
| Range, maximum |  | 1,060 | 1,220 | 1,963 |  |
| Range, with standard fuel - at 205 mph and 45min reserve |  | 693 | 797 | 1,283 | at 9,000ft. |
| Range, with standard fuel - at 205 mph, no reserve |  | 824 | 948 | 1,526 | at 9,000ft. |
| Range, with standard fuel - 45% power, 163mph |  | 1,034 | 1,190 | 1,915 | at 15,000ft. 45 min. reserve. |
| Range, economy cruising power, max payload |  |  |  |  |  |
|  |  | kts | mph | km/hr |  |
| Manoeuvring | Va | 142 | 163 | 262 | True Indicated |
| Maximum structural cruising | Vno | 200 | 230 | 370 | True Indicated |
| Never Exceed | Vne | 250 | 288 | 463 | True Indicated |
| Flaps extended - Half |  | 130 | 150 | 241 | True Indicated |
| Flaps extended - Full |  | 118 | 136 | 219 | True Indicated |
| Landing gear extended | Vle | 156 | 180 | 290 | True Indicated |
| Maximum Operating speed | Vmo |  |  |  |  |
| Maximum level speed |  | 187 | 215 | 346 | T.A.S. at sea level |
| Maximum cruising speed |  |  |  |  | T.A.S. |
| Economy cruising speed - 55% power |  | 176 | 202 | 325 | T.A.S. at 10,000ft / 3,048m |
| Economy cruising speed - 75% power |  | 176 | 203 | 327 | T.A.S. at 9,000ft / 2,745m |
| Takeoff speed | V2 | 83 | 95 | 153 |  |
| Landing speed |  | 77 | 89 | 143 |  |
| Best rate-of-climb speed (Vy) | Vy | 96 | 110 | 177 |  |
| Best Angle of Climb speed (Vx) | Vx |  |  |  |  |
| Minimum Single-Engine Control speed | Vmc | 65 | 75 | 121 |  |
| Stall Speed (Clean) (Vsi) | Vs1 | 68 | 78 | 126 | C.A.S. |
| Stall Speed (Dirty) (Vso) | Vso | 59 | 68 | 109 | C.A.S. |

| Engines (1): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics |  | Piston, air cooled, Fuel Injected, with 6 horizontally Opposed cylinders |  |  |  |
| Manufacturer |  | Lycoming |  |  |  |
| Engine Type |  | IO-540 |  |  |  |
| Engine Variant |  | -E1A5 | -E1B5 |  |  |
| Type Certificate Number |  | 1E4 | 1E4 |  |  |
| Type Certificate Date |  | June 15th 1962 | March 29th 1966 |  |  |
| Applicable under STC No. |  | n/a | n/a |  |  |
| Maximum continuous HP |  | 290 | 290 |  |  |
| Maximum continuous RPM |  | 2,575 | 2,575 |  |  |
| Takeoff HP (5 minutes) |  | 290 | 290 |  |  |
| Takeoff RPM |  | 2,575 | 2,575 |  |  |
| Weight (lb.) |  | 411 | 412 |  |  |
| Reduction Gear Ratio |  | n/a | n/a |  |  |
| Supercharging Ratio |  | n/a | n/a |  |  |
| Bore & Stroke (inches) |  | 5.125 x 4.375 | 5.125 x 4.375 |  |  |
| Displacement (cu.in.) |  | 541.5 | 541.5 |  |  |
| Compression Ratio |  | 8.7 : 1 | 8.7 : 1 |  |  |
| Injector Make & Model |  | Bendix (now P.A.C.) RS10B1 |  |  |  |
| Notes: | The -E1A5 has internal piston cooling oil jets, thereby increasing maximum heat rejected to oil to 1150 BTU per minute. |  |  |  |  |
|  | The -E1B5 is the same as -E1A5 except that it has TCM S6LN-1208 & 1209 magnetos. |  |  |  |  |
|  | No Models 500S left the factory with the -E1A5 engine, although it has been used as a replacement on a few Shrikes, with Rajay Turbochargers installed. |  |  |  |  |

| Engines (2): |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics |  | Piston, air cooled, Fuel Injected. |  |  |  |
|  |  | with 8 horizontally Opposed cylinders. |  | Turbocharged with 6 horizontally Opposed cylinders. |  |
| Manufacturer |  | Lycoming |  | Lycoming |  |
| Engine Type |  | IO-720 |  | TIO-540 |  |
| Engine Variant |  | -B1B | -B1BD | -J2B | -J2BD |
| Type Certificate Number |  | 1E15 | 1E15 | E14EA | E14EA |
| Type Certificate Date |  | February 22nd 1971 | December 30th 1976 | September 9th 1981 | April 7th 1972 |
| Applicable under STC No. |  | SA2478SW and SA2691WE |  | SA5969NM | SA5969NM |
| Maximum continuous HP |  | 400 | 400 | 350 | 350 |
| Maximum continuous RPM |  | 2,650 | 2,650 | 2,575 | 2,575 |
| Takeoff HP (5 minutes) |  | 400 | 400 | 350 | 350 |
| Takeoff RPM |  | 2,650 | 2,650 | 2,575 | 2,575 |
| Weight (lb.) |  | 557 | 557 | 558 | 558 |
| Reduction Gear Ratio |  | n/a | n/a | n/a | n/a |
| Supercharging Ratio |  | n/a | n/a | n/a | n/a |
| Bore & Stroke (inches) |  | 5.125 x 4.375 | 5.125 x 4.375 | 5.125 x 4.375 | 5.125 x 4.375 |
| Displacement (cu.in.) |  | 722 | 722 | 541.5 | 541.5 |
| Compression Ratio |  | 8.70 : 1 | 8.70 : 1 | 7.30 : 1 | 7.30 : 1 |
| Injector Make & Model |  | Bendix (now P.A.C.) RSA-10ED1 |  | Bendix (now P.A.C.) RSA-10ED1 |  |
| Notes: | The -B1BD has dual Magnetos |  |  |  |  |
|  | The -J2BD has dual Magnetos |  |  |  |  |

| Propellers: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Characteristics |  | Constant speed, hydraulic, with feathering and unfeathering features. |  |  |  |  |  |  |
|  |  |  |  | with Counterweighted blades. |  |  |  |  |
| Manufacturer |  | Hartzell |  |  |  |  |  |  |
| Hub Model(s) |  | HC-A3XK | HC-A3VK | HC-C3YR |  |  | HC-C4YR |  |
| Hub Variant |  | -2 | -2A | -2 & -2U | -2F & -2UF |  |  | -2UF |
| Type Certificate Number |  | P-913 | P6EA | P25EA |  |  | P52GL |  |
| Type Certificate Date |  | Feb 15th 1960 | Aug 15th 1962 | Apr 1st 1966 |  |  | Sep 21st 1977 |  |
| Applicable under STC No. |  | n/a |  |  |  |  |  |  |
| Hub Material |  | Alloy Steel |  | Aluminum Alloy |  |  |  |  |
| Blade Model(s) |  | 8433-4 | V8433-4 | C8468-6R | FC8468-6R | FC6660K |  |  |
|  |  |  | V8433A-4 |  | FC-8468D-6R |  |  |  |
|  |  |  | V8433N-4R |  | FC-8468B-6R |  |  |  |
| Blade Material |  | Aluminum Alloy |  |  |  |  |  |  |
| Number of Blades |  | 3 |  | 3 |  |  | 4 |  |
| Diameter (inches) |  | 80 |  | 80 |  |  | 70 |  |
| Diameter (meters) |  | 2.032 |  | 2.032 |  |  | 1.778 |  |
| Cut-off permitted |  | none |  | none |  |  |  |  |
| Prop tip ground clearance (inches) |  | 27.34 (Drawings 5010221 & J010247 = 27.30") |  |  |  |  | 32.34 |  |
| Prop tip fuselage clearance (inches) |  | 12.40 (Drawings 5010221 & J010247 = 11.00") |  |  |  |  | 17.40 |  |
| Distance between propeller centers |  | 153" |  |  |  |  |  |  |
| Pitch setting at 30” Station - Low |  | 12.5o to 13o |  | 12.5o to 13o |  |  |  |  |
| - High |  |  |  |  |  |  |  |  |
| - Feathered |  | 85.5o |  | 85.5o |  |  |  |  |
| Maximum Hub & Blade weight (lb.) |  | 94 |  | 84 |  |  | 117 |  |
| HP - Maximum continuous |  | 380 | 400 | 400 |  |  | 400 |  |
| RPM - Maximum continuous |  | 2,700 | 2,650 | 2,700 |  |  | 2,650 |  |
| HP - Maximum Takeoff |  | 380 | 400 | 400 |  |  | 400 |  |
| RPM - Maximum Takeoff |  | 2,700 | 2,650 | 2,700 |  |  | 2,650 |  |
| Notes: | ‘F’ suffix in Hub designation denotes modified pitch change system. |  |  |  |  |  |  |  |
|  | ‘U’ suffix in Hub designation denotes added feather assist spring. |  |  |  |  |  |  |  |
|  | ‘C’ prefix in Blade designation denotes counterweighted blades. |  |  |  |  |  |  |  |
|  | ‘F’ prefix in Blade designation denotes a large pitch change knob. |  |  |  |  |  |  |  |
|  | ‘R’ suffix in Blade designation denotes specifically rounded tip for cut-off diameter. |  |  |  |  |  |  |  |
|  | ‘B’ or ‘K’ suffix in Blade designation denotes de-icing boots. |  |  |  |  |  |  |  |
|  | ‘D’ suffix in Blade designation denotes dimensional modification from original design. |  |  |  |  |  |  |  |
|  | The HC-C3YR-2 hub is wrongly listed on the TCDS as “HC-3YR-2”. |  |  |  |  |  |  |  |

| Engine & Propeller combinations: |  |  |
| --- | --- | --- |
| Engines | Propellers (Hub / Blade) | Factory (Unit numbers) or other |
| Lycoming IO-540-E1A5 | Hartzell HC-C3YR-2 / (not quoted) | Field (no doubt C8468-6R) |
|  | Hartzell HC-C3YR-2UF / FC8468-6R | Field |
| Lycoming IO-540-E1B5 | Hartzell HC-A3VK-2A / V8433-4 | Field (Bob Hoover) |
|  | Hartzell HC-A3VK-2A / V8433A-4 | Factory (1755-1 to 3053) |
|  | Hartzell HC-A3XK-2 / 8433-4 | Field (Bob Hoover) |
|  | Hartzell HC-C3YR-2 / C8468-6R | Factory (3054 to 3123) |
|  | Hartzell HC-C3YR-2F / FC8468-6R | Factory (3124 to 3278 + 3300) |
|  | Hartzell HC-C3YR-2U / C8468-6R | Field |
|  | Hartzell HC-C3YR-2UF / FC8468-6R | Factory (3279 - 3282, 3290 - 3299 & 3301 - 3323) |
|  | Hartzell HC-C3YR-2UF / FC8468B-6R | Field |
| Lycoming TIO-540-J2B | Hartzell HC-C3YR-2UF / FC8468-6R | STC SA5969NM |
| Lycoming IO-540-J2BD | Hartzell HC-C3YR-2UF / FC8468D-6R | STC SA5969NM |
|  | Hartzell HC-C4YR-2UF / FC6660K |  |
| Lycoming IO-720-B1B | Hartzell HC-A3VK-2A / V8433N-4R | STC SA2478SW & SA2691WE |
| Lycoming IO-720-B1BD | Hartzell HC-A3VK-2A / V8433N-4R | STC SA2478SW |

Production:

I’ve shown below, a table outlining the total production. It does not, however, necessarily reflect the year in which they were built, being compiled from the first Airworthiness Certificate date for each serial number. Additionally, if an Export Certificate was issued within a month of Certification, that has been used instead of the ‘domestic’ Certificate.

| Certificated under | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Experimental |  |  | 1 |  | 1 |  | 2 |
| 6A1 & Preliminary Revision dated 15Mar68 | 2 |  |  |  |  |  | 2 |
| 6A1 & preliminary Revision dated 08Apr68 | 2 |  |  |  |  |  | 2 |
| Revision 27 | 19 |  |  |  |  |  | 19 |
| Revision 27 & Proposed Revision dated 19Sep68 |  | 15 |  |  |  |  | 15 |
| Revision 27 & Proposed Revision dated 17Jun69 |  | 2 |  |  |  |  | 2 |
| Revision 28 & Proposed Revision dated 13Aug69 |  | 19 | 8 |  |  |  | 27 |
| Revision 29 |  |  | 4 |  |  |  | 4 |
| Revision 29 & Proposed Revision dated 12Aug70 |  |  | 1 |  |  |  | 1 |
| Revision 30 |  |  | 1 | 1 |  |  | 2 |
| Revision 31 & Proposed Revision dated 24Feb71 |  |  |  | 1 |  |  | 1 |
| Revision 31 & Proposed Revision dated 04May71 |  |  |  | 3 |  |  | 3 |
| Revision 31 & Proposed Revision dated 18Oct71 |  |  |  | 3 | 2 |  | 5 |
| Revision 31 & Proposed Revision dated 09Mar72 |  |  |  |  | 10 |  | 10 |
| Revision 31 & Proposed Revision dated 04Aug72 |  |  |  |  | 1 |  | 1 |
| Revision 32 |  |  |  |  | 1 |  | 1 |
| Revision 32 & Proposed Revision dated 02Feb73 |  |  |  |  |  | 13 | 13 |
| Export | 7 | 10 | 4 | 9 | 14 | 20 | 64 |
| Totals | 30 | 46 | 19 | 17 | 29 | 33 | 174 |

| Certificated under | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Experimental |  |  | 1 |  |  |  | 1 |
| Revision 32 & Proposed Revision dated 20Sep73 | 2 |  |  |  |  |  | 2 |
| Revision 32 & Proposed Revision dated 16Apr74 | 1 |  |  |  |  |  | 1 |
| Revision 32 & Proposed Revision dated 15May74 | 1 |  |  |  |  |  | 1 |
| Revision 32 & Proposed Revision dated 12Jul74 | 1 |  |  |  |  |  | 1 |
| Revision 33 & Proposed Revision dated 15Nov74 | 3 | 1 |  |  |  |  | 4 |
| Revision 33 & Proposed Revision dated 16Jan75 |  | 8 |  |  |  |  | 8 |
| Revision 33 & Proposed Revision dated 08May75 |  | 3 |  |  |  |  | 3 |
| Revision 33 & Proposed Revision dated 05Aug75 |  | 4 |  |  |  |  | 4 |
| Revision 34 |  | 3 | 3 |  |  |  | 6 |
| Revision 35 |  |  | 1 |  |  |  | 1 |
| Revision 35 & Proposed Revision dated 24Aug76 |  |  | 2 | 3 |  |  | 5 |
| Revision 35 & Proposed Revision dated 20Apr77 |  |  |  | 6 | 11 | 2 | 19 |
| Revision 36 |  |  |  |  |  | 1 | 1 |
| Export | 41 | 26 | 9 | 3 | 4 | 2 | 85 |
| Totals | 49 | 45 | 16 | 12 | 15 | 5 | 142 |

Total: 1968 through 1973	174

Total: 1974 through 1979	142

Total Production:	316

Model 500S - Current Fleet Status:

The following figures can, of course, change on virtually a daily basis, but will reflect the latest position that I am aware of. Naturally, they will be updated with each Fact File Revision for this Model.

Note that ‘WFU’ stands for Withdrawn From Use, and comprises those Commanders which have been scrapped or are being parted out. In this instance, it also includes 2 examples in the U.S. which have been reported as stolen, and 8 examples in ‘Registered elsewhere in the World’, where Registration has been cancelled for an as yet undetermined reason.

|  | Current | Destroyed | WFU | Total |
| --- | --- | --- | --- | --- |
| Registered in the USA | 94 | 29 | 5 | 128 |
| Registered elsewhere in the World | 140 | 34 | 14 | 188 |
| Totals: | 234 | 63 | 19 | 316 |
| As percentage of the Total for the Model | 74.05 | 19.94 | 6.01 |  |

Identification Features:

The following tables are intended as a guide to the features, which would have differentiated one Model apart from another as they left the factory.

My attempt has been to group the Models firstly by nacelle type, ‘Bathtub’, ‘Speedline’ and ‘Turbo’, and then group them by ‘short-wing or long-wing’ or ‘short-body or long body’, but only Models in the same nacelle group and wing or body group are itemised in this Fact File.

Of course, over the years many STC’s have been incorporated into the various Models since they left the factory. For instance, Gary Gadbery’s Aircenter Inc have made many a 500B into a Shrike by installing the Miller nose, cockpit eyebrow windows, the ‘stinger’ tail cone and a square vertical tip. Nowadays, it is becoming more and more difficult to positively tell one Model apart from another. Winglets and ‘Dash 10’ engines installed on the earlier Turbos compound the problem. In reality, it is only the older ‘Bathtub’ Models which remain relatively easy to identify with any degree of accuracy.

Blank spaces really indicate a ‘No’, but to include that in the tables only serves to ‘clutter’ the picture.

| Nacelle Group: Speedline |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Short-body Models (1) |  |  |  |
| Feature | 500A | 560F | 680F | 680F(P) |
| Standard length nose | Yes | Yes | Yes | Yes |
| Lengthened nose |  |  |  |  |
| Nose lights | Yes | Yes | Yes | Yes |
| Number of propeller blades | 2 | 3 | 3 | 3 |
| Number of nacelle front intakes |  | 3 | 3 | 3 |
| Eyebrow windows |  |  |  |  |
| Strengthened or no DV window |  |  |  | Yes |
| Dorsal air scoop |  |  |  | Yes |
| Fin-mounted aerials |  |  |  |  |
| Control surfaces - beaded | Yes | Yes | Yes | Yes |
| Control surfaces - smooth |  |  |  |  |
| Same features- need defining |  | YES |  |  |

| Nacelle Group: Speedline |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Short-body Models (2) |  |  |  |  |  |  |
| Feature | 500B (i) | 500B (ii) | 500U | 500S (i) | 500S (ii) | 500S (iii) | 500S (iv) |
| Standard length nose | Yes | Yes | Yes | Yes |  |  |  |
| Lengthened nose |  |  |  |  | Yes | Yes | Yes |
| Nose lights | Yes | Yes | Yes | Yes | Yes |  |  |
| Number of propeller blades | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of nacelle front intakes | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Eyebrow windows |  |  |  |  |  |  | Yes |
| Strengthened or no DV window |  |  |  |  |  |  |  |
| Dorsal air scoop |  |  |  |  |  |  |  |
| Fin-mounted aerials |  |  |  |  | Yes/no |  | Yes |
| Fin cap - rounded | Yes | Yes | Yes | Yes | Yes | Yes |  |
| Fin cap - squared |  |  |  |  |  |  | Yes |
| Control surfaces - beaded | Yes |  |  |  |  |  |  |
| Control surfaces - smooth |  | Yes | Yes | Yes | Yes | Yes | Yes |
| Applicable to s/ns | all up to 1545-195 | 1551-197 & subs. | all | all up to 3050 | 500S (1) modified | 3050 to 3075 | 3076 to 3323 |
| Same features- need defining |  | YES |  |  |  |  |  |

| Current Airworthiness Directives |  |  |  |  |
| --- | --- | --- | --- | --- |
| AD number |  | Rev. no. | Subject | see Note No. |
| 59-16-01 |  |  | Fuel line routing | 1 |
| 61-14-01 |  |  | Engine mount inspection | 2 |
| 75-12-09 |  | R2 | Inadvertent structural failure | 3 |
| 87-21-07 |  | R1 | Fuel filler openings | 4 |
| 94-04-11 |  |  | Wing front spar lower cap | 5 |
| 94-04-13 |  |  | Wing front spar lower cap | 6 |
| 94-04-14 |  |  | Wing front spar lower cap | 7 |
| 98-07-17 |  |  | Flap system cable | 8 |
| 98-08-19 |  |  | Inspect wing to fuselage attach points for cracks | 9 |
| 98-08-25 |  | R1 | NLG drag link bolt | 10 |
| 98-20-34 |  |  | AFM - Limitations Section - Icing | 11 |
| ** Notes ** |  |  |  |  |
| Note No. | Remarks (per heading of the Regulatory Information) |  |  |  |
| 1 | Applies to all Models 500, 520, 560, 560A, 560E, 680, 680E, 720 Aircraft, Serial Numbers 231 through 690. |  |  |  |
| 2 | Applies to all Model 500 Aircraft. |  |  |  |
| 3 | Applies to Models 500, 500A, 500B, 500S, 500U, 520, 560, 560A, 560E, 560F, 680, 680E, 680F, 680F(P), and 720, through Serial Number 3323 airplanes certificated in all categories. |  |  |  |
| 4 | Applicability: Models 500, 500A, 500B, 500U, 520, 560, 560A, 560E, 560F, 680, 680E, 680F, 680F(P), 680FL(P), and 720 Serial Numbers (s/n 1 through 1854); and Models 500S (s/n 1755 through 3323), 680FL (s/n 1261 through 1853), 685 (s/n 12001 through 12066), and 700 (s/n 70001 through 70032) airplanes equipped with reciprocating engines, certificated in any category. |  |  |  |
| 5 | Applicability: The following Model and serial number airplanes, certificated in any category: 500S (s/n 3314 through 3323); 690B (s/n 11480 through 11566). |  |  |  |
| 6 | Applicability: All serial numbers of the following model airplanes, certificated in any category: 500; 500A; 500B; 500S; 500U; 520; 560; 560A; 560E; 560F; 680; 680E; 680F; 680FL(P); 680FL; 680T; 680V; 680W; 681; 720. |  |  |  |
| 7 | Applicability: The following model and serial number airplanes that do not have the wing front spar lower cap replaced in accordance with the procedures of one of the three modifications specified in paragraph (b) of this AD, certificated in any category: 500U, 680FL, 680FL(P), and 680W (s/n 1731 through 1854); 500S (s/n 1755 through 3323); 681 (s/n 6001 through 6072); 685 (s/n 12000 through 12066); 690, 690A, and 690B (s/n 11001 though 11566). |  |  |  |
| 8 | Applicability: The following airplane models (all serial numbers), certificated in any category: 500; 500A; 500B; 500S; 500U; 520; 560; 560A; 560E; 560F; 680; 680E; 680F; 680FL; 680FL(P); 680FP; 680T; 680V; 680W; 681; 685; 690; 690A; 690B; 690C; 690D; 695; 695A; 695B; 720. |  |  |  |
| 9 | Applicability: Models 500, 500A, 500B, 500S, 500U, 520, 560, 560A, 560E, 560F, 680, 680E, 680F, 680FL, 680FLP, 680FP, 680T, 680V, 680W, 681, 685, 690, 690A, 690B, 690C, 690D, 695, 695A, 695B and 720 airplanes, all serial numbers, certificated in any category. |  |  |  |
| 10 | Applicability: The following model and serial number airplanes, certificated in any category: 500S (s/n 3185, 3228, 3230, 3262, and 3291). 500U (s/n 1765); 680F (s/n 1195); 681 (s/n 6027); 680V (s/n 1677); 690 (s/n 11035, 11053, 11068, and 11074); 690A (s/n 11111, 11134, 11146, 11153, 11173, 11177, 11205, 11215, 11237, 11249, 11271, 11273, and 11282); 690B (s/n 11360, 11382, 11409, 11424, 11451, 11455, 11463, 11491, 11513, 11521, 11535, 11536, 11539, and 11566); 690C (s/n 11638, 11643, 11676, 11689, and 11719); 690D (s/n 15041. 695 95010, 95033, 95044, and 95066); 695A (s/n 96010, 96041, 96056, and 96061). |  |  |  |
| 11 | Applicability: Models 500, 500A, 500B, 500S, 500U, 520, 560, 560A, 560E, 560F, 680, 680E, 680FL(P), 680T, 680V, 680W, 681, 685, 690, 690A, 690B, 690C, 690D, 695, 695A, 695B, and 720 airplanes (all serial numbers), certificated in any category. |  |  |  |
| Note: | These AD’s are as they appear under the Model 500S, from the FAA website. However, ‘Note Numbers’ 1 & 2 shouldn’t be there, as they apply to the straight Model 500. Indeed, the date of the AD’s tends to confirm this! |  |  |  |